Immunohistochemical staining for paraffin tissue

準備工作

配製fresh wash buffer. (1X PBS + 0.1% Tween 20)

(實例:2L = 200ml 10X PBS + 1800ml ddH2O + 2ml Tween 20)

(注意:未加Tween 20的1X PBS可滅菌消毒，若滅完有減少，請補已滅菌的ddH2O；吸取Tween 20時，因其較為黏稠，為避免沾黏，請吸取液體表面，慢吸慢吐，以免有體積上的誤差，若用blue tip, 可割去部分tip尖端，如此較易吸取Tween 20) (要利用磁石攪拌均勻)

注意事項

- 記得要有negative control及positive control.(注意specimen與一級抗體及一級抗體與二級抗體之間的關係)

- 請用鉛筆標示玻片.

- 使用的slide最好是coated slide，與組織的黏合效果較好.

- 須考量Trilogy與一級抗體間是否合用?

- The detection system must be matched to the species of primary antibody.(目前Core Lab所使用的super sensitive polymer-HRP detection system only for Rabbit、Mouse and Goat)

Deparaffinization and Rehydration

- Collect sectioned slides on slides coated with poly-L-lysine or silane.

- Slides置於鐵架上，整個置於鐵盤或塑膠盤上，以防止溶解之paraffin掉到烘箱，再放入烘箱內heat the slides at 58C for 2hrs.(此步驟的目的為融蠟；記得玻片朝上)(若是skin檢體，則為3hrs)

Antigen Retrieval (此部份的目的為脫蠟、脫水及恢復抗原的立體結構，曝露出來讓抗體才能作嵌合)

- 以20X稀釋Trilogy，即19份ddH2O加1份的Trilogy，混合均勻.(於耐熱的塑膠玻片缸中)，從烘箱拿出slides後，要立刻放入Slides於此缸中。

- 電高壓鍋方法:選高壓模式，溫度設為95℃，時間設為10或15分.(若是skin檢體，則時間設為5分)，若反應結果不明顯，可將溫度設為120℃反應10分鐘。

(注意:打開鍋蓋前，請確認壓力值已降至0)

- 直接隔水放冷，放冷須超過5分.

(處理的片數較少時，舊Trilogy可再使用三次)

Blocking of Endogenous Peroxidase

(Endogenous peroxidase or pseudoperoxidase activity found in hemoglobin, myoglobin, cytochrome, and catalase as well as in neutrophils, monocytes, and eosinophils, may yield false-positive results.)

- Rinse slides 2~3 times with distilled water (盡量將殘蠟洗掉) and tap off excess water. Carefully wipe around specimen.

- 準備一個黑色的濕盒並於盒底擺放沾濕的擦手紙數張.(注意:可沾濕一點，以防實驗過程乾掉)

- 將slides放入濕盒內.

- Mark the specimen using DAKO pen.

- Apply H2O2 (3%) to cover specimen for 5min.

(實例:0.8ml 10X PBS + 7.2ml ddH2O + 0.75ml 35%H2O2 or 30% H2O2及ddH2O以1:9的比例稀釋) (注意:3% H2O2要每次實驗前配製及35% H2O2要分裝使用，以免產生氧化)

- 蓋上濕盒的盒蓋.

- Rinse slides gently with distilled water for 5min and place in fresh wash buffer (PBS + 0.1% Tween 20) bath for 5min.

Ab Incubation and Color Development

- Tap off excess liquid and wipe slides as before. 將slides放入濕盒內.

- Dilute Ab in Ab diluent (0.05M Tris-HCl pH 7.2, 0.1% Tween 20, 1% BSA, 15 mM NaN3; DAKO S3022).( NaN3的作用是當防腐劑)

- Apply enough primary antibody or negative control to cover specimen and蓋上濕盒的盒蓋. Incubate at 37℃ for 2hr.

    - 注意:primary antibody要蓋滿組織

    - 室溫, 1-2 hr或 4℃, overnight 亦為常用之抗體作用時間, 但若採用室溫時, 要注意冬天及夏天之室溫可能有相當差距, 此差距可能影響結果。

    - 注意不同抗體之保存溫度不同, 檢體需保存在冷凍狀態, 則冷凍前先分成小包裝再冷凍，避免多次解凍。

- Rinse slides gently with wash buffer (可用前一次的) for 5min and place in fresh wash buffer bath for 5min.

- Immediately tap off excess buffer and wipe slides as before. 將slides放入濕盒內.

- Apply enough Super Enhancer reagent to cover specimen and蓋上濕盒的盒蓋. Incubate at RT for 20 min. Rinse slides as step 4. Apply enough Poly-HRP reagent to cover specimen and蓋上濕盒的盒蓋. Incubate at RT for 30 min.

- Rinse slides as step 4.

- Wipe slides as before. 將slides放入濕盒內.

- Apply enough substrate buffer-DAB+ chromogen solution(50:1;加入後記得要混合均勻，因為DAB會沈澱) to cover specimen and蓋上濕盒的盒蓋. Incubate at RT for 5 min.(注意:需避光)

(實例:1ml = 1ml substrate buffer + 20μl DAB+ chromogen or 1ml substrate buffer + 1 drop(38μl) DAB chremogen)

(Buffered solution containing hydrogen peroxide and preservative.

3,3'-diaminobenzidine tetrahydrochloride in organic solvent. The colour of this reagent may vary from strong violet to colourless without having any influence on the performance of the kit)

- Rinse slides gently with distilled water and immerse slides in a bath of Mayer's hematoxylin for 5 sec to 5 min (建議:30sec), depending the strength of hematoxylin used.

(注意:若是使用回收的Mayer's hematoxylin，請先用濾紙過濾；若染的顏色不夠深，則可再染一次，若染的顏色太深，則可用water洗褪)

- Rinse gently in a bath of water. (注意:要以輕注入的方式，wash for 1 min)

- 使用Non-Xylene浸潤玻片1min使玻片檢體透明。(用過的Non-Xylene收集至廢液桶)

- Place the slides at room temperature for 10min.(於有抽氣的空間內為最佳)

- Mounted and coverslipped slides with mounting medium(請用油性的，勿用水性的).

(注意:蓋上蓋玻片後，要用鑷子輕壓將氣泡趕出)

- Store slides in the dark (注意:用黑色且乾燥的濕盒即可，蓋上盒蓋並留一指縫的空隙，再放於有抽氣的空間內) at room temperature. (20-25C)

- 長期保存: dark at room temperature. (20-25C)

Notes:

- Do not allow tissues to dry during the staining.

- Use moisture box for staining.

- 若有背景太強的問題，則可於原H2O2 blocking後新增加一個blocking的步驟:Thermo Ultra V Block solution / 1:8 / 5 mins.