- General Annexin V Staining Procedure ( BD Bioscience)

- Solutions

- 10X Binding Buffer (cat. no. 556454): 0.1 M HEPES, pH 7.4; 1.4 M NaCl; 25 mM CaCl 2. Dilute to 1X prior to use.

- Propidium Iodide (PI, cat. no. 556463).

- Annexin V-FITC (cat. no. 556420, 556419)

- 1X PBS Buffer (cat. no. 554781): 8 g NaCl, 0.2 g KCl, 1.44 g Na2HPO 4 • 7H20, 0.24 g KH2PO4 , H20 to 1 liter. Adjust pH to 7.2, autoclave and store at RT.

- Procedure

- Wash cells twice with cold PBS and then resuspend cells in 1X Binding Buffer at a concentration of ~1 x 106 cells/ml.

- Transfer 100 µl of the solution (~1 x 105 cells) to a 12 x 75mm tubes

- Add Annexin V (5 ul) and PI (2 ul)

*The optimal amount of PI may range between 2–10 µl/test depending on cell type and experimental system. 2 µl/test is the recommended starting amount.

- Gently mix the cells and incubate for 15 min at RT in the dark.

- Add 400 µl of 1X Binding Buffer to each tube. Analyze by flow cytometry as soon as possible (within 1 hr).

The following controls are used to set up compensation and quadrants:

- Unstained cells

- Cells stained with Annexin V-FITC alone (no PI)

- Cells stained with PI alone (no Annexin V-FITC)